| RTKLIB ver. 2.3.0 Manual |  |
| --- | --- |

2009/12/17

Contents

1	Overview	1

2	System Requirement	3

3	Instructions	4

3.1	Installation	4

3.2	Real-time Positioning with RTKNAVI	5

3.3	Configure Input, Output and Log Streams for RTKNAVI	16

3.4	Post-Processing Analysis with RTKPOST	20

3.5	Configure Positioning Options for RTKNAVI and RTKPOST	24

3.6	Convert Receiver Raw Data to RINEX with RTKCONV	34

3.7	View and Plot Solutions and Observation Data with RTKPLOT	37

3.8	NTRIP Source Table Browser	45

3.9	Use Console APs of RTKLIB	47

4	Develop and Link user AP with RTKLIB	48

Appendix A  Console AP Commands	49

A.1	RTKRCV	49

A.2	RNX2RTKP	52

A.3	POS2KML	54

A.4	CONVBIN	55

A.5	SBSPOS	57

A.6	SBSDUMP	58

Appendix B  File Formats	59

B.1	Positioning Solution File	59

B.2	SBAS Log File	62

B.3	Solution Status File	63

Appendix C  Library APIs (Application Program Interfaces)	66

# 1	Overview

RTKLIB is an open source program package for RTK-GPS/GNSS. RTKLIB consists of a simple and portable program library and several application programs (APs) utilizing the library. The program library of RTKLIB provides:

- Matrix and vector functions

- Time and string functions

- Coordinates transformation and geoid model

- Navigation processing

- Positioning models (troposphere, ionosphere, antenna PCV)

- SBAS DGPS/DGNSS correction

- Single point positioning

- Carrier-based and code-based relative positioning

- OTF integer ambiguity resolution

- Receiver raw binary data input

- Positioning solution/NMEA input/output

- RINEX observation data/navigation message input/output

- Precise ephemeris input

- Stream data communication library

- NTRIP (Networked Transport of RTCM via Internet Protocol) library

- RTK-GPS/GNSS positioning server

- RTCM 2.3 and 3.0/3.1 message handling

The GUI and console (command line) APs of RTKLIB includes:

- Real-time positioning (RTKNAVI, RTKRCV)

- Post-processing baseline analysis (RTKPOST, RNX2RTKP)

- Communication utility (STRSVR, STR2STR)

- Plot graph of positioning solution and observation data (RTKPLOT)

- RINEX converter of receiver raw data log (RTKCONV, CONVBIN)

- Other positioning utilities

All of the executable binary APs for Windows are included in the package as well as whole source programs of the library and the APs.

The receiver have to output raw measurement data of pseudorange/carrier-phase and satellite ephemerides. NRTK (Network RTK) service supporting RTCM 2 or 3 can also be used for the base-station. Please refer Release Notes for supported receivers and data messages. The post-processing baseline analysis can process RINEX 2.10 or 2.11 observation and navigation messages (GPS and GLONASS) supported by many receivers. Version 2.3.0 supports satellite navigation system of GPS, GLONASS and SBAS. Future version will support other GNSSs such as Galileo and QZSS.

# 2	System Requirement

The library and its APIs of RTKLIB use standard ANSI C (89) functions. The library internally use standard socket and pthread APIs for Linux/UNIX or winsock and WIN32 thread for Windows. By setting compiler option -DLAPACK or -DMKL, the library uses LAPACK/BLAS or Intel MKL for fast matrix computation.

The console APs are also written in standard C and use these APIs. They can be built on many environments like gcc on Linux or MAC OS X. The GUI APs are written in C++ and utilize Borland VCL for the GUI library.

All of the binary (console and GUI) APs in the package were built by free edition Borland Turbo C++ 2006 (http://www.turboexplorer.com) * on Windows. The binary APs were tested on Windows XP pro SP3 32bit and Windows Vista Home 64bit.

* Free edition Borland Turbo C++ 2006 is no longer available (2009/11).

# 3	Instructions

## 3.1	Installation

- Extract the program package rtklib_<ver>.zip or rtklib_<ver>_bin.zip to appropriate directory (<ver> indicates the version number). The RTKLIB directory structure is as follows.

rtklib_<ver>

\src			: Source programs of RTKLIB libraries *

\rcv			: Source programs depending on GPS/GNSS receivers *

\bin			: Executable binary APs and DLLs for Windows

\data			: Sample data for APs

\app			: Build environment for APs *

\rtknavi		: RTKNAVI (GUI) *

\strsvr			: STRSVR (GUI) *

\rtkpost		: RTKPOST (GUI) *

\rtkpost_mkl		: RTKPOST_MKL (GUI ) *

\rtkplot		: RTKPLOT (GUI) *

\rtkconv		: RTKCONV (GUI) *

\srctblbrows 		: NTRIP source table browser (GUI) *

\rtkrcv			: RTKRCV (console) *

\rnx2rtkp		: RNX2RTKP (console) *

\pos2kml		: POS2KML (console) *

\convbin		: CONVBIN (console) *

\sbsdump 		: SBSDUMP (console) *

\sbspos			: SBSPOS  (console) *

\str2str		: STR2STR (console) *

\appcmn			: Common routines for GUI APs *

\icon			: Icon data for GUI APs *

\mkl			: Intel MKL libraries for Borland environment *

\test			: Test program and data *

\util			: Utilities *

\doc			: Document files

* Not included in the package rtklib_<ver>_bin.zip

- Create the shot-cuts of the GUI AP executable binaries in rtklib_<ver>\bin. To execute console APs, add <install dir>\rtklib_<ver>\bin to the command path.

## 3.2	Real-time Positioning with RTKNAVI

Real-time positioning AP RTKNAVI inputs raw observation data of GPS/GNSS receivers and execute navigation processing in real-time. By setting the positioning mode to Kinematic and configuring the rover and the base station receiver data inputs, RTK-GPS/GNSS positioning is enabled with OTF (On-the-fly) integer ambiguity resolution.

- Execute the binary AP file rtklib_<ver>\bin\rtknavi.exe. (Double-click the icon or full in the  path in the command console) You can see the main window of RTKNAVI.

Time Display

Solution Display

RTK Monitor

Button

Input/Output/Log

Stream

Status/Settings

Signal Level/

Satellite Visibility Display

Message Area

Save Log Button

Display Switch Buttons

Operation

Buttons

- The following figure shows the data flow of RTKNAVI. You have to set Input Streams, Output Streams (optional) and Log Streams (optional) for real-time positioning.

Output Streams (O)

RTKNAVI

(3) Output Solution 1

(1) Input Rover

(2) Input Base/NRTK

(4) Output Solution 2

Input Streams (I)

GPS/GNSS Receivers

(5) Log Rover

(6) Log Base/NRTK

Log Streams (L)

- For real-time positioning with RTKNAVI, you have to input the raw observation data and satellite ephemerides from the GPS/GNSS receivers. To set the input stream, push the button   I   upper center in the main window. You can see "Input Streams" dialog.

- Check and set the stream type of the rover and the base-station/NRTK (network RTK) in the dialog. If you set the positioning mode option "Single", the input stream for the base-station/NRTK is not required. The stream types can be selected from the following options.

(a) Serial		: Input data from a serial port (RS232C or USB)
(b) TCP Client		: Connect to a TCP server and input data via the TCP connection
(c) TCP Server		: Accept a TCP client connection and input data via the TCP connection
(d) NTRIP Client	: Connect to a NTRIP server (caster) and input data via the NTRIP. 
			 NRTK (network RTK) server supporting NTRIP and RTCM 2/3 can also
			 be used for the base-station via Internet.
(e) File		: Input data from a log file.

You have to select the stream data format from the following options with the pull down menu under "Format". Refer Release Notes for the detailed supported messages.

(a) RTCM2		: RTCM 2.3
(b) RTCM3		: RTCM 3.0 or 3.1
(c) NovAtel OEM4/V	: NovAtel OEM4/V binary format
(d) NovAtel OEM3	: NovAtel OEM3 (Millennium) binary format
(e) u-blox		: u-blox LEA-4T/5T binary format
(f) Superstar II		: NovAtel Superstar II binary format
(g) Hemisphere		: Hemisphere Crescent/Eclipse binary format
(h) SkyTraq		: SkyTraq S1315F binary format

- If you select Serial as the stream type, push  ...  button under "Opt" label to set the options of port selection, bit-rate, byte size, parity, number of stop bits and flow control with "Serial Options" dialog.

- In case of selecting Serial as the stream type, you can configure the startup and shutdown commands to be sent to the GPS/GNSS receiver. Push   ...  button under "Cmd" label. Fill in the commands in the text fields in "Serial Commands" dialog. If you do not check "Commands at startup" or "Commands at shutdown", the startup or shutdown command is not sent to the receiver. You can also load the commands from a command file by pushing  Load...  button or save the commands to a command file with  Save...  button. The command file is just a text file including startup commands and shutdown commands separated by a line starting with "@". The sample command files for some typical receivers are found in the directory rtklib_<ver>/data.

- The line starting with "!" is treated as a receiver binary command. The following commands can be used for u-blox and SkyTraq receivers. Refer receivers' manuals for details.

!UBX ... : u-blox LEA-4T/5T command

!UBX CFG-PRT   portid res0 res1 mode baudrate inmask outmask flags

!UBX CFG-USB   vendid prodid res1 res2 power flags vstr pstr serino

!UBX CFG-MSG   msgid rate0 rate1 rate2 rate3

!UBX CFG-NMEA  filter version numsv flags

!UBX CFG-RATE  meas nav time

!UBX CFG-CFG   clear_mask save_mask load_mask

!UBX CFG-TP    interval length status time_ref res adelay rdelay udelay

!UBX CFG-NAV2  ...

!UBX CFG-DAT   maja flat dx dy dz rotx roty rotz scale

!UBX CFG-INF   protocolid res0 res1 mask0 mask1 mask2 mask3

!UBX CFG-RST   navbbr reset res

!UBX CFG-RXM   gpsmode lpmode

!UBX CFG-ANT   flags pins

!UBX CFG-FXN   flags treacq tacq treacqoff tacqoff ton toff res basetow

!UBX CFG-SBAS  mode usage maxsbas res scanmode

!UBX CFG-LIC   key0 key1 key2 key3 key4 key5

!UBX CFG-TM    intid rate flags

!UBX CFG-TM2   ch res0 res1 rate flags

!UBX CFG-TMODE tmode posx posy posz posvar svinmindur svinvarlimit

!UBX CFG-EKF   ...

!STQ ... : SkyTraq S1315F binary command

!STQ RESTART  [arg...] system restart

!STQ CFG-SERI [arg...] configure serial port property

!STQ CFG-FMT  [arg...] configure output message format

!STQ CFG-RATE [arg...] configure binary measurement output rates

!WAIT time : wait for time (ms)

- If you select TCP Client or TCP Server as the stream type, you can set the options of TCP server address (for TCP Client only) and port number with "TCP Client Options" or "TCP Server Options" dialog. If you select "TCP Server", multiple TCP client connections are allowed.

- If you select NTRIP Client as the stream type, you can set the NTRIP client options of NTRIP caster host address, port number, mountpoint of NTRIP caster, user-ID and password with "NTRIP Client Options" dialog.

- If you select File as the stream type, input the file path to the text field Input File Paths. Fill in the path directly or select a file with the file selection dialog by pushing  ...  button. The input file should be a  receiver raw data log. You can set the replay speed and the start time offset of the log file in Time field (To use the feature, you have to record the log with the time-tag file.)

- In case of using NRTK (network RTK) service, which requires NMEA GPGGA messages to select reference station or to get virtual reference station position, select  the message content with the pull down menu at Transmit NMEA GPGGA to Base/NRTK. If you select Latitude/Longitude to send fixed position, fill in the latitude and longitude for NMEA GPGGA message in degree (minus means south or west).

- To output of the positioning solution of RTKNAVI, set the output streams. To set the output streams, push the button   O   upper right in the main window. You can see "Output Streams" dialog.
Check and set the stream type of solution in the dialog. You can configure two independent output streams. You can select the stream type out of Serial, TCP Client, TCP Server, NTRIP Server and File. The options are similar to the input streams. You also have to select the following output format options. The time and latitude/longitude formats in output messages can be configured by the positioning options.
(a) Lat/Lon/Height		: Latitude, longitude and height
(b) X/Y/Z-ECEF			: X/Y/Z components in ECEF frame
(c) E/N/U-Baseline		: E/N/U components of the baseline
(d) NMEA-183			: NMEA GPRMC, GPGGA, GPGSV (1Hz only)

- If you select "File" as the output stream type, you can include some keywords in the file path to be replaced by date or time. Push   ?   button to show the keyword replacement in file paths.

- To output an input stream as a path-through log, set the log streams. To configure the log streams, push the button   L   upper right in the main window. You can see "Log Streams" dialog. The settings  are similar to the output streams. If you want to replay the log file as an input stream later, you have to check "Time-Tag" option and output the time tag file simultaneously. The output path of the time tag file is output file path + .tag. The keyword replacements in the file paths are the same as "Output Streams" dialog.

- To configure the positioning options, push  Options...  button and set the options in "Options" dialog. For details of the positioning options, refer 3.5 Configure Positioning Options for RTKNAVI and RTKPOST.

- Push   Start   button. The status of each streams are shown on the upper right indicators. From the left, they show the stream/processing status of Input Rover, Input Base/NRTK, the positioning process, Output Solution 1, Output Solution 2, Log Rover and Log Base/NRTK. Gray represents not used, Orange means waiting for the connection, Deep-green means connected or running, Light-green means data active (input, output or processing) and Red means a communication error occurs. Some status messages also are shown in the lower center message area in the main window. To stop the positioning process in RTKNAVI push  Stop  button.

- After the input observation data and ephemerides are completed and valid, RTKNAVI computes the positioning solution and display it in the solution display left in the main window with the solution status (FIX, FLOAT, DGPS, SBAS or SINGLE), E/N/U or X/Y/Z components of the standard deviation, Age (age of differential), Ratio (ratio factor of ambiguity validation) and # of Sat (number of valid satellites). To switch the format in the solution display, push  button upper right corner. You can switch the solution to Lat/Lon/Height (degree/minute/second), Lat/Lon/Height (degree), X/Y/Z-ECEF (m), E/N/U-Baseline (m), Pitch/Yaw/Length-Baseline (deg, m), alternatively.

- To switch the format in the time display, push  GPST  button upper center in the main window. You can switch the time system to GPST, UTC, JST and GPST (GPS week/TOW), alternatively.

- In the status display right in the main window, observation SNR status or visible satellites in skyplot are shown. By pushing  button upper right corner of the status display, you can switch the contents to Rover : Base/NRTK SNR, Rover SNR, Rover Skyplot, Base/NRTK Skyplot and Baseline plot,  alternatively.

- By pushing  Map...  button, you can display "RTK Map" window indicating the current position of the rover receiver on the map. To expand or shrink the scale of the map, push  or　　button upper in the window. To fix the current receiver position at the map center, use    button. The button  is also used to switch on/off of the receiver trajectory. To show the vertical position graph at the bottom of the window, push  button. To close the window, push  Close  button.

- On the "RTK Map" window, by pushing  button and  ...  at upper left corner, you can see the "Point List" dialog.  With the dialog, you can load, save, add and delete the way-points as the list form. By pushing  Add  button and editing the point name, the current receiver position can be added to the way-point list. The positions of the way-points as well as the base-station are also shown on the map when   button down.

- The positioning solutions are recorded in the internal buffer simultaneously. You can save the internal solution buffer to the file by pushing  ...   below the solution display. The size of the solution buffer and the saved solution log can be configured with "Options" dialog.

- By pushing the  button lower left in the main window, you can see "RTK Monitor" window. With the window, you can monitor the internal status of RTKNAVI. You can select the following contents with the upper left pull down menu. Multiple "RTK Monitor" windows are allowed. To close the window, push  Close  button.

(a) RTK		: General status of the internal positioning process
(b) Satellites		: Status of each satellites
(c) States		: State vector values of the estimation filter
(d) Covariance		: Covariance matrix of the estimation filter 
(e) Obs Data		: Input observation data. RCV=1 means rover and 2 means base-station
(f) GPS Nav		: GPS navigation messages
(g) GLONASS Nav	: GLONASS navigation messages

(h) GEO Nav		: SBAS satellites navigation messages
(i) Streams		: Status of input, output and log streams
(j) SBAS Msgs		: HEX dump of input SBAS messages
(k) SBAS Long		: SBAS long term satellite corrections
(l) SBAS Iono		: SBAS ionospheric delay corrections
(m) SBAS Fast		: SBAS fast corrections
(n) RTCM		: Status of RTCM 2 or 3 messages
(o) RTCM DGPS		: RTCM DGPS corrections
(p) Input Rover		: Dump of Input Rover stream
(q) Input Base/NRTK	: Dump of Input Base/NRTK stream
(r) Solution 1/2		: Dump of Output Solution 1/2 stream
(s) Error/Warning	: Error or warning messages

- By pushing   button lower right in the main window, you can minimize the main window as an icon in the task-tray of Windows desktop. To restore the main window, double-click the task-tray-icon or click right-button on the task-tray-icon and select menu "Main Window...".

## 3.3	Configure Input, Output and Log Streams for RTKNAVI

For relative positioning like RTK-GPS/GNSS, the rover receiver and the base station receiver are usually placed on the separated sites. In other cases, user might use the positioning result at the remote site far from these receivers. To interconnect these sites, user has to establish data communication links. To facilitate easy setup of these communication links, RTKLIB provides the communication utility STRSVR, with which user can configure input and output data stream via these communication links. STRSVR also has the function of relay or split the data stream for real-time positioning with RTKNAVI.

For example, to receive observation data of a remote base station at a rover receiver site and to get RTK-GPS solution, user can place a remote PC installing STRSVR connecting to the base station receiver and can configure STRSVR to send data to the rover site. The following examples show the typical applications of RTKNAVI and SVRSVR.

- Single-point positioning and output solutions to a file

PC

GPS/GNSS Receiver

RTKNAVI

Serial

File

(1) Input Rover=Serial

(3) Output Solution 1=File

- Single point positioning, output solutions to a serial device, log data to a file

PC

RTKNAVI

Serial

Serial

Log

GPS/GNSS Receiver

(1) Input Rover=Serial

(3) Output Solution 1=Serial

(5) Log Rover=File

- RTK-GPS/GNSS, input the rover and base-station data from two serial devices

PC

RTKNAVI

File

Rover

Receiver

Serial

Base Station

Receiver

Serial

(1) Input Rover=Serial

(2) Input Base Station=Serial

(3) Output Solution 1=File

- RTK-GPS/GNSS, input rover data from a serial port and input base-station data from a remote receiver via WiFi network.

Base Station

Receiver

Rover

Receiver

RTKNAVI

Serial

PC

Serial

STRSVR

PC

File

TCP

Server

TCP

Client

(1) Input Rover=Serial

(2) Input Base Station=TCP Client

(3) Output Solution 1=File

(0) Input =Serial

(1) Output 1=TCP Server

- RTK-GPS/GNSS, input rover data from a serial port and input base station data via mobile phone Internet connection

Base Station

Receiver

Mobile　Phone　N/W

Serial

PC

Serial

STRSVR

PC

Rover

Receiver

RTKNAVI

File

Internet

Mobile

Phone

Terminal

(1) Input Rover=Serial

(2) Input Base Station=TCP Client

(3) Output Solution 1=File

(0) Input =Serial

(1) Output 1=TCP Server

- RTK-GPS/GNSS, input data from a serial port and input base station data via NTRIP caster on Internet

Ntrip

Caster

PC

PC

Rover

GPS Receiver

RTKNAVI

Serial

File

Base Station

GPS Receiver

Serial

STRSVR

Internet

(1) Input Rover=Serial

(2) Input Base Station=NTRIP Client

(3) Output Solution 1=File

(0) Input =Serial

(1) Output 1=NTRIP Server

- RTK-GPS/GNSS with NRTK (Network RTK) service via Internet

Reference Station

Reference Station

Reference Station

Reference Station

Rover

GPS Receiver

RTKNAVI

Serial

(1) Input Rover=Serial

(2) Input Base Station=NTRIP Client

(3) Output Solution 1=File

File

PC

NTRIP Caster

Internet

NRTK provider

(8) Multiple RTK-GPS/GNSS with single NRTK service

(1) Input =NTRIP Client

(2) Output=TCP Server

NTRIP Caster

Internet

NRTK provider

Reference Station

Reference Station

Reference Station

Reference Station

STRSVR

PC

LAN

RTKNAVI

PC

Rover

Receiver 1

RTKNAVI

PC

Rover

Receiver 2

RTKNAVI

PC

Rover

Receiver 3

RTKNAVI

PC

Rover

Receiver 4

(1) Input Rover=Serial

(2) Input Base Station=TCP Client

The following instructions are for the operation of STRSVR.

- Execute the binary AP file rtklib_<ver>\bin\strsvr.exe. You can see the main window of STRSVR.

- To configure the input stream, select the stream type with pull down menu at "(0) Input". Selectable stream types are Serial, TCP Client, TCP Server, NTRIP Client and File. The stream options or the startup/shutdown command can be set as well as Input Streams for RTKNAVI.

- To configure the output streams, select the stream type with pull down menu at (1) Output, (2) Output or (3) Output. The setting for the output streams are same as Output Streams or Log Streams for RTKNAVI.

- Push  Start  button in the main window. The communication status is shown in the message area lower center of the main window. Status indicators left side of the main window also shows the communication status. The indicator colors means: Orange: waiting connection, Dark-Green: connected, Light-Green: data active, Red: error. Total data amount (bytes) and data rate (bps) of the input and output streams are also shown in right side. To stop the communication, push  Stop  button.

- By pushing  Options...  button, you can set the communication options with "Options" dialog. To send NMEA GPGGA message to the server connected the input stream, check "NMEA Request Cycle" and set the request cycle (ms) and latitude/longitude in the messages.

3.4	Post-Processing Analysis with RTKPOST

RTKLIB contains a post processing analysis AP RTKPOST. RTKPOST inputs the standard RINEX 2.10 or 2.11 observation data and navigation message files (GPS and GLONASS) and can computes the positioning solutions by carrier-based relative positioning.

- Execute the binary AP file rtklib_<ver>\bin\rtkpost.exe. You can see the main window of RTKPOST. You can execute the binary AP file rtklib_<ver>\bin\rtkpost_mkl.exe instead. RTKPOST_MKL is a faster version of RTKPOST, which link Intel MKL library for fast matrix computation.

- Input the RINEX observation data file path of the rover receiver in the text field Observation Data. Fill in the file path or select a file using the file selection dialog shown by pushing  ...  button. You also have to input the path of RINEX navigation message files of GPS and GLONASS in Navigation Messages field. If you leave it blank, the observation data file path with the extension replaced by .nav and .gnav (.obs) or .yyn and .yyg (.yyo) is used for the navigation message files of GPS and GLONASS. You can use the compressed file by GZIP (.gz), COMPRESS (.z) or Hatanaka-Compression (.yyd) for the RINEX observation data or navigation messages. If a wild-card (*) is included in the file path,  the wild-card expanded multiple files are used.

- If you process the RINEX data in the positioning mode of DGPS/DGNSS, Kinematic, Static,  Moving-Base or Fixed, input the RINEX observation data file path of the base-station receiver as well as the rover observation data.

- Input the output file path in the text field Output File.

- Push  Options...  button to set the processing options. For detailed options, refer 3.5 Configure Positioning Options for RTKNAVI and RTKPOST. You can set the start time or end time by checking and setting Time Start (GPST) or Time End (GPST) field in the main window. You also set the time interval by checking and setting Interval field. With the  ?  button, the input time in GPS time can be converted to UTC, GPS Week/TOW, Day of Year, Day of Week, Time of Day and Leap Seconds.

- If you check both of Time Start and Time End, you can check Unit for multiple session analysis. If Unit checked and set the Unit, the analysis session is separated to multiple sessions for the Unit (hr). To avoid overwriting the output to the same file, the output file path has to contain the keyword replaced according to the session time.  For details of the keyword replacement in the input or output file paths, refer 3.5 Configure Positioning Options for RTKNAVI and RTKPOST.

- Push  Execute   button to start the analysis.  The processing status is shown in the status message field lower center in the main window. When you see "done" message here, the analysis is completed. If you want to stop the processing on the way, push  Abort  button.

- After completing the analysis, by pushing  View...  button, you can display the content of the output file by Text Viewer. You can reload the output file by pushing   button in Text Viewer window. To close the window, push  Close  button.

- By pushing  View...  button, you can also plot the result with RTKPLOT. Refer 3.7 View and Plot Solutions and Observation Data with RTKPLOT for details.

- By pushing  To KML...  button, the output file can be converted to Google Earth KML file with "Google Earth Converter" dialog. Set or select the options and push  Convert  button in the dialog.

- With   button in the main window, you can view and plot the input observation data RTKPLOT. You can also display the contents of the input files with Text Viewer by pushing  button.

## 3.5	Configure Positioning Options for RTKNAVI and RTKPOST

By pushing  Options...  button in the main windows of RTKNAVI or RTKPOST, you can set the positioning options. Selectable or changeable positioning options are as follows.

(1) Setting 1

| Item | Descriptions | Notes |
| --- | --- | --- |
| Positioning Mode | Set positioning mode - Single : Single point positioning or SBAS DGPS - DGPS/DGNSS : Code-based differential GPS - Static : Carrier-based Static positioning - Kinematic: Carrier-based Kinematic positioning - Moving-Base: Moving baseline - Fixed: Rover receiver position is fixed |  |
| Frequencies | Set used carrier frequencies - L1 : L1 only - L1+L2 : L1 and L2 |  |
| Solution Type | Set solution type - Forward: Forward filter solution - Backward: Backward filter solution * - Combined: Smoother combined solution with forward and backward filter solutions * | * RTKPOST only |
| Elevation Mask | Set elevation mask angle (deg) |  |
| SNR Mask | Set SNR mask (dBHz) |  |
| Receiver Dynamics | Set the dynamics model of the rover receiver. - OFF: Dynamics is not used - ON: Receiver velocity and acceleration are estimated. The receiver position is predicted with the estimated velocity and acceleration. | Only effective in DGPS/DGNSS or Kinematic mode |
| Ionosphere Estimation | Set whether ionospheric parameters (vertical ionospheric delay for a satellite) are estimated or not. For long baseline analysis, ionosphere estimation is effective to suppress ionosphere delay effects. - OFF : Not estimate ionospheric parameters - ON : Estimate ionospheric parameters |  |
| Troposphere Estimation | Set whether tropospheric parameters (zenith total delay at rover and base-station positions) are estimated or not. - OFF : Not estimate tropospheric parameters - ON : Estimate tropospheric parameters |  |
| Satellite Ephemeris | Set the type of satellite ephemeris. - Broadcast : Use broadcast ephemeris - Precise : Use precise ephemeris * If using precise ephemeris, set the precise ephemeris file path in Files settings | * RTKPOST only |
| Excluded Satellites (PRN) | Set the excluded satellites for positioning. Fill in the PRN numbers of the satellites separated by spaces. |  |
| Navi System | Check used navigation satellite systems. If not checked, satellites of the system are not used for positioning. - GPS - GLONASS - Galileo * - QZSS * - SBAS ** | * Not supported yet ** RTKNAVI only |

(2) Setting 2

| Item | Descriptions | Notes |
| --- | --- | --- |
| Integer Ambiguity Resolution | Set the strategy of integer ambiguity resolution - OFF : No ambiguity resolution - Continuous : Continuously static integer ambiguities are estimated and resolved - Instantaneous : Integer ambiguity is estimated and resolved by epoch-by-epoch basis - Fix and Hold : Continuously static integer ambiguities are estimated and resolved. If the validation OK, the ambiguities are held as the resolved values. |  |
| Validation Threshold of AR | Set the integer ambiguity validation threshold for ratio-test, which uses the ratio of squared residuals of the best integer vector to the second-best vector. | Default value: 3.0 |
| Lock Count to Fix Ambiguity | Set the lock count to fix integer ambiguity. If the lock count is less than the value, the ambiguity is excluded from the fixed integer vector. | Default value: 0 |
| Outage Count to Reset Ambiguity | Set the outage count to reset ambiguity. If the data outage count is over the value, the estimated ambiguity is reset to the initial value. | Default value: 5 |
| Elevation Mask for AR | Set the elevation mask angle (deg) to fix integer ambiguity. If the elevation angle is less than the value, the ambiguity is excluded from the fixed integer vector. | Default value: 0 |
| Slip Threshold of LG-Difference | Set the cycle-slip threshold (m) of geometry-free LC carrier-phase difference between epochs. | Default value: 0.05 |
| Max Age of Differential | Set the maximum value of age of differential (s) between the rover and the base station. | Default value: 30 |
| Reject Threshold of Innovation | Set the reject threshold of innovation (pre-fit residual) (m). If the innovation is over the value, the observable is excluded for estimation as an outlier. | Default value: 30 |
| Number of Iteration | Set the number of iteration in the measurement update of the estimation filter. If the baseline length is very short like 1 m, the iteration is effective to handle the nonlinearity of measurement equation. | Default value: 1 |
| Baseline Length Constraint | If Moving-Base mode, check and set the constraint of the baseline length. Fill in the length in m and the standard deviation (m) of the constraint. |  |

(3) Output

| Item | Descriptions | Notes |
| --- | --- | --- |
| Solution Format | Set the output solution format. - Lat/Lon/Height : Latitude, longitude and height - X/Y/Z-ECEF : X/Y/Z components of ECEF coordinates - E/N/U-Baseline: E/N/U components of baseline vector - NMEA0183 : NMEA GPRMC, GPGGA, GPGSV | For RTKNAVI, specify options as Output Streams setting. |
| Output Header | Set whether the header is output or not. | Not applicable to NMEA |
| Output Processing Options | Set whether the processing options are output or not. | RTKPOST only Not applicable to NMEA |
| Time Format | Set the format of time - ssssssss.sss GPST : GPS week and TOW (time of week) - hh:mm:ss GPST : yyyy/mm/dd hh:mm:ss GPST - hh:mm:ss UTC : yyyy/mm/dd hh:mm:ss UTC - hh:mm:ss JST : yyyy/mm/dd hh:mm:ss JST | Not applicable to NMEA |
| # of Decimals | Set number of decimals in the time format | Not applicable to NMEA |
| Latitude/Longitude Format | Set the formats of latitude and longitude if the solution format is set to Lat/Lon/Height. - ddd.dddddddd : Degree - ddd mm ss.sss : Degree minute second | Not applicable to NMEA |
| Field Separator | Set the separator for fields. | Not applicable to NMEA |
| Datum | Set the datum if the solution format option is set to Lat/Lon/Height. - WGS84 : WGS84 datum - Tokyo : Tokyo datum (current version supports only WGS84) |  |
| Height | Set the type of height. - Ellipsoidal : Ellipsoidal height - Geodetic : Geodetic height |  |
| Geoid Model | Set the geoid model if the Height option is set to Geodetic. - Internal: Internal geoid model - EGM96-BE (15") : EGM96 (15" x 15" grid) *1 - EGM2008-SE (2.5") : EGM2008 (2.5 x 2.5" grid) *2 - EGM2008-SE (1"): EGM2008 (1 x 1" grid) *2 - GSI2000 (1x1.5"): GSI2000 (1x1.5" grid) *3 If using external geoid model, specify the geoid file path in "Files" tab. |  |
| Output Solution Status | Set the output level of the solution status file. The solution status file contains estimated states and residuals. The solution status file is created in the current directory (RTKNAVI) or in the output file directory (RTKPOST). |  |
| Output Debug Trace | Set the output level of debug trace file. If setting OFF, any debug trace file is not output. The debug trace file is created in the current directory (RTKNAVI) or in the output file directory (RTKPOST). |  |

*1 WW15MGH.DAC (http://earth-info.nga.mil/GandG/wgs84/gravitymod/egm96/binary/binarygeoid.html )

*2 Und_min1x1_egm2008_isw=82_WGS84_TideFree_SE,  Und_min2.5x2.5_egm2008_isw=82_WGS84_TideFree_SE

(http://earth-info.nga.mil/GandG/wgs84/gravitymod/egm2008/egm08_wgs84.html )

*3 gsigeome.ver4 (http://vldb.gsi.go.jp/sokuchi/geoid/download/down.html)

(4) Statistics

| Item | Descriptions | Notes |
| --- | --- | --- |
| Measurement Errors (1-sigma) |  |  |
| Code/Carrier-Phase Error Rate | Set the ratio of standard deviations of pseudorange errors to carrier-phase errors. | Default value: 100 |
| Carrier-Phase Error | Set the base term of carrier-phase error standard deviation (m). | Default value: 0.003 |
| Carrier-Phase Error/sinEl | Set the elevation dependent term of carrier-phase error standard deviation (m/sin(el)). | Default value: 0.003 |
| Carrier-Phase Error/Baseline | Set the baseline-length dependent term of carrier-phase error standard deviation (m/10km). | Default value: 0 |
| Doppler Frequency | Set the standard deviation of Doppler errors (Hz) (Current version does not use the value) | Default value: 1 |
| Process Noises (1-sigma/sqrt(s)) |  |  |
| Receiver Accel Horiz/Vertical | Set the process noise standard deviation of the receiver acceleration as the horizontal or vertical component. (m/s2/sqrt(s)). If Receiver Dynamics is set to OFF, they are not used. | Default value: 1 and 0.1 |
| Carrier-Phase Bias | Set the process noise standard deviation of carrier-phase bias (ambiguity) (cycle/sqrt(s)). | Default value: 1E-4 |
| Vertical Ionospheric Delay | Set the process noise standard deviation of vertical ionospheric delay per 10 km baseline (m/sqrt(s)). | Default value: 1E-3 |
| Zenith Tropospheric Delay | Set the process noise standard deviation of zenith tropospheric delay (m/sqrt(s)). | Default value: 1E-4 |
| Satellite Clock Stability | Set the satellite clock stability(s/s). The value is used for interpolation of base-station observables. | Default value: 5E-12 |

(5) Positions

| Item | Descriptions | Notes |
| --- | --- | --- |
| Rover |  |  |
| Lat/Lon/Height (deg/m) | Set the position of the rover antenna if the rover antenna is fixed. (current version does not support the option) |  |
| Antenna Type | Select the type of the rover antenna. To select the antenna type, set the Receiver Antenna PCV File path in Files. |  |
| Delta-E/N/U | Set the delta position of the rover antenna as the E/N/U offsets of ARP (antenna reference point) position with refer to the marker (m). |  |
| Base Station |  |  |
| Lat/Lon/Height (deg/m) | Set the position of the base-station antenna. - Lat/Lon/Height (deg/m): Latitude/longitude/height in degree and m - Lat/Lon/Height (dms/m): Latitude/longitude/height in degree/minute/second and m - X/Y/Z-ECEF (m): X/Y/Z components in ECEF frame. - RTCM Station Position: Use the antenna position included in RTCM messages * - Average of Single-Pos: Use the average of single point solutions ** - Get from Position File: Use the position in the position file. The station is searched by using the head 4-character ID of the rover observation data file path. ** - RINEX Header Position: Use the approximate position in RINEX OBS header. ** | Height is specified as ellipsoidal height * RTKNAVI only ** RTKPOST only |
| Antenna Type | Select the type of the base-station antenna. To select the antenna type, set Receiver Antenna PCV File in Files. |  |
| Delta-E/N/U | Set the delta position of the base-station antenna as E/N/U offsets of ARP position with refer to the marker (m). |  |

If you set Station Position File in "Files" tab, you can select the potion of the rover or the base-station antenna from the station list in "Stations" dialog by pushing  ...  button.

(6) Files

| Item | Descriptions | Notes |
| --- | --- | --- |
| Satellite Precise Ephemeris SP3 | If you select precise ephemeris as Satellite Ephemeris in Setting 1, input the precise ephemeris file path (NGS SP3 format). If a wild-card (*) is included in the file path, wild-card-expanded multiple files are used. | RTKPOST only |
| Satellite Antenna PCV File ANTEX | If you input the precise ephemeris file, input the ANTEX antenna parameters file path for the satellite antenna PCV (phase center variation) correction. Usually use latest igs05.atx file provided by IGS. An example of the ANTEX file is found at rtklib_<ver>\data\igs05.atx. | RTKPOST only |
| Receiver Antenna PCV File NGS | If you apply the receiver antenna phase center offset and PCV correction, input NGS type antenna parameters file path. An example of the antenna parameter file is found at rtklib_<ver>\data\ngs_abs.pcv. |  |
| Station Position File | Input the station position file path to retrieve the position from the station list. The station position file is a text file which contains the multiple lines. Each line represents a record for a station. A record contains: - Latitude (deg) - Longitude (deg) - Ellipsoidal height (m) - Station ID - Station name separated by spaces. The line starting "%" is treated as the comment line. An example station position file is found at rtklib_<ver>\data\stations.pos. |  |
| Geoid Data File | Input the file path of the geoid data file if selecting the external model as Geoid Model. |  |
| Google Earth Exe File | Input the execution file path of Google Earth. |  |

(6) Misc (RTKNAVI)

| Item | Descriptions | Notes |
| --- | --- | --- |
| Processing Cycle | Set the processing cycle time of in ms. Usually set 100 ms or less value. |  |
| Timeout/ Re-Connect Interval | Set the timeout and re-connect interval for TCP client and NTRIP client connections in ms. If the timeout time expired without sever response, RTKNAVI retries to connect to server after waiting for the re-connect interval. |  |
| NMEA Transmission Cycle | Set the NMEA GPGGA transmission cycle to NRTK server in ms. |  |
| Input Buffer Size | Set the internal input message buffer size in bytes. Usually set it to 32768 or more. |  |
| Solution Buffer/Log Size | Set the internal solution buffer size and log size in epochs. To increase the length of the receiver trajectory on "RTK Map", increase the solution buffer size. |  |
| Navigation Message Selection | Select navigation message to be used. - Rover: In rover receiver stream - Base Station: In base station receiver stream - Both: In both streams |  |
| DGPS/DGNSS Corrections | If you want to enable SBAS DGPS/DGNSS corrections, select SBAS. If not, select OFF. |  |
| SBAS Satellite Selection | If SBAS DGPS correction enabled, input SBAS satellite PRN number to be used. If you input 0, all available SBAS satellites are used. |  |
| SBAS Corrections | If SBAS DGPS correction enabled, check enabled corrections of long term, ionospheric, fast corrections and ranging signal. |  |
| Solution Font | Select the font of the solution display in the main window. |  |

(7) Misc (RTKPOST)

| Item | Descriptions | Notes |
| --- | --- | --- |
| Time Interpolation of Base Station Observation Data | Check it to enable time interpolation of base station data. If checked, the base station data are linearly interpolated to the rover epoch and DD (double-difference) is made with them. If not checked, nearest epoch of base station data is used for DD. |  |
| Keyword Replacement in File Path | For batch processing with multiple input files or multiple sessions, you can set input file paths or output file path containing the following keywords. %Y, %y, %m, %d, %n, %W, %D, %h, %H, %r, %b The keywords are replaced by the proper values or expanded for multiple session analysis. To enable time keywords, set Time Start, Time End and Unit (optional) in the main window. For keywords %r, %b , input Rover List or Base Station List below. |  |
| Rover List | Input the rover ID list to replace keyword %r in input and output file paths. The line starting with "#" is treated as a comment. |  |
| Base Station List | Input the base station ID list to replace keyword %b in input and output file paths. The line starting with "#" is treated as a comment. |  |

## 3.6	Convert Receiver Raw Data to RINEX with RTKCONV

RINEX (Receiver Independent Exchange Format) is a standard GPS/GNSS data format supported by many receivers or GPS/GNSS post-processing analysis software. RTKLIB post-processing analysis AP RTKPOST can also handle RINEX data files as inputs. For preparing RINEX files, RTKLIB provides the converter AP RTKCONV, which translates receiver raw data to RINEX OBS (observation data), RINEX NAV (GPS navigation messages) and RINEX GNAV (GLONASS navigation messages). RTKCONV can also extract SBAS messages from the receiver raw data and output the SBAS log file.

The supported RINEX version is 2.10 or 2.11. RINEX 3.0 will be supported by the future version. Refer the Appendix B.2 SBAS Log File for the SBAS log file format.

- Execute the binary AP file rtklib_<ver>\bin\rtkconv.exe. You can see the main window of RTKCONV.

- Input the receiver raw data file path to the text field Log File. Fill in the file path directly or select the file with the file selection dialog by pushing  ...  button. You can also drag and drop the icon of the raw data file to the main window of RTKCONV. Supported receiver raw data formats are as follows. Refer Release Notes for detailed supported messages.

(a) RTCM2		: RTCM 2.3
(b) RTCM3		: RTCM 3.0 or 3.1
(c) NovAtel OEM4/V	: NovAtel OEM4/V binary format
(d) NovAtel OEM3	: NovAtel OEM3 (Millennium) binary format
(e) u-blox		: u-blox LEA-4T/5T binary format
(f) Superstar II		: NovAtel Superstar II binary format
(g) Hemisphere		: Hemisphere Crescent/Eclipse binary format
(h) SkyTraq		: SkyTraq S1315F binary format

- Select the format with the pull down menu Format. If you select "Auto", RTKCONV recognizes the file format according to the following file extensions.

(a) RTCM2		: .rtcm2
(b) RTCM3		: .rtcm3
(c) NovAtel OEM4/V	: .gps
(d) u-blox		: .ubx
(e) Superstar II		: .log
(f) Hemisphere		: .bin
(g) SkyTraq		: .stq

- Input the output paths of RINEX OBS (observation data), RINEX NAV (GPS navigation messages), RINEX GNAV (GLONASS navigation messages) and SBAS Log files. Fill in the file path directly or select the file with the file selection dialog by pushing  ...  button. If you do not check the checkbox left, the file is not output.

- You can set the start time or end time optionally by checking and setting Time Start (GPST) or Time End (GPST) field upper in the main window. You can also set the time interval option by checking and setting the field Interval.

- You can push  Options...  button to configure RINEX options. Set the options for RINEX headers, navigation systems, observation types or frequencies with "Options" dialog .

- Push  Convert  button to start converting the receiver raw data to RINEX and SBAS log files. If you want to stop the conversion on the way, push  Abort  button. The status is displayed in the message area lower center in the main window. The message O=nnn means the number of converted observation data (epochs). The message N=nnn, S=nnn and E=nnn means the number of navigation messages, SBAS messages and errors, respectively.

- After finishing the conversion, you can see the observation data plot by pushing  Plot...  button with RTKPLOT. Refer 3.7 Plot and View Solutions and Observation Data for details. You can also view the output file with Text Viewer by pushing  button.

## 3.7	View and Plot Solutions and Observation Data with RTKPLOT

RTKLIB contains the AP RTKPLOT to view and plot the positionig solutions by RTKLIB or RINEX observation data with graphical user interface.

- Execute the binary AP file rtklib_<ver>\bin\rtkplot.exe. You can see the main window of RTKPLOT. By pushing  Plot...  button or some buttons of RTKPOST and RTKCONV, RTKPLOT is also executed.

Reload Button

Time  Scroll-bar

Plot Type

Selection

Tool Buttons

Status Bar

Tool Bar

Plot Area

- Execute the menu "File" - " Open Solution 1" and select the solution file with the file selection dialog. The input solution file can be RTKLIB solution format or NMEA-0183. If the file format is NMEA, the file has to contain at least GPRMC and GPSGGA sentences. If the solution file is valid, the receiver ground track is plot in the window on the map. The color of the marks, lines and grid in the plot can be changed with menu "Edit" - "Options". The status bar at the bottom of the main window also shows the time range, the number of solution epochs (N=nnnn), the baseline length (B=0.0-x.xkm), the number and percentage of each quality solutions (Q=1:nnn(pp%), 2:nnn(pp%),...). The quality flag Q and the marker color means: 1: Fixed, 2: Float, 4: DGPS, 5: Single (the colors are changeable with the plot options). To screen the marks by the quality flag Q, select the second pull down menu in the Tool Bar. By drag and drop of the solution file icon to the main window of RTKPLOT, you can also read and plot the solution file.

- By dragging the mouse with the left button down on the plot, you can drag the map up, down, left and right .  You also change the scale of the map by dragging the mouse up or down with the right button or by rotating the scroll wheel of the mouse.

- By selecting the pull down menu right in Tool Bar, you can switch the plot to E/N/U components of receiver position (Position), E/N/U components of  receiver velocity (Velocity) or E/N/U components of receiver acceleration (Accel). You can drag the X/Y-axis with the left-button dragging and change the scale with the right-button dragging at the X/Y-axis area.

- By selecting the pull down menu right in Tool Bar, you can switch the plot to NSat/Age/Ratio (number of valid satellites, age of differential, ratio factor of ambiguity validation). If you set Output Solution Status option to Residuals, you can show the residuals plot by selecting L1 Residuals or L2 Residuals. In residuals plot mode, you can select a satellite with right pull down menu as well as all satellites. In the residuals plot of carrier-phase, the red lines indicate cycle slips and gray lines indicate parity unknown flags (That mean the half-cycle ambiguities in carrier-phase are not resolved).

- By opetating tool buttons in Tool Bar, you can center the current position with , adjust the scale of X-axis with , adjust the scale of Y-axis with , display the current position as a large mark with ,  fix the current position at the center with  , start animation with  and stop animation with . You can also slide Time Scroll Bar to change the epoch of the current position. To reload the solution file, execute the menu "File" - "Reload" or push  button in Tool Bar.

- To plot multiple solution file, execute the menu "File" - "Open Solutions-2" and select file with the file selection dialog. You can switch the plot on/off of the solution 1 and 2 with   1    2   buttons in Tool Bar. To Plot the difference of the solution 1 and the solution 2, push  1-2  button in Tool Bar.

- To set the time range and time interval of the solutions, execute the menu "Edit" - "Time Span/Interval" and check and set the Time Start, Time End and Interval field in "Time Span/Interval" dialog.

- To plot the RINEX observation data, execute menu "File" -- "Open Obs Data" and select the RINEX observation and navigation message files. You can select multiple RINEX files. If you do not select the RINEX navigation message file, RTKPOST reads the observation data file paths with the extension replaced by .nav and .gnav (.obs) or .yyn and .yyg (.yyo) as the GPS and GLONASS navigation data. If you want read the RINEX navigation message file separately, execute the menu "File" - "Open Nav Messages". If the input files can be read properly, you can see the satellite visibility or the observation data availability plot. You can use left-button or right-button drag to change the time span. You can also use some tool buttons in Tool Bar as well as  for the solution plot.

- By selecting the pull down menu in Tool Bar, you can switch the plot to satellite visibility in skyplot (SkyPlot) or plot of number of visible satellites and DOP (Sats/DOP).

- To configure the plotting options, execute menu "Edit" - "Options..." and set the options with the following "Options" dialog.

| Item | Descriptions | Notes |
| --- | --- | --- |
| Time Format | Select time format. |  |
| Show Statistics | Set whether statistics are shown or not. |  |
| Cycle-Slip | Set whether cycle-slip position is shown or not in observation data display. Cycle-slips are shown as red vertical lines in the observation data plot. |  |
| Parity Unknown | Set whether parity unknown status is shown or not in observation data display. Parity unknown epochs are shown as gray vertical lines in the observation data plot. |  |
| Ephemeris | Set whether ephemeris status is shown or not in observation data display. Ephemerides are shown as the grey line under the observation data plot. Gray dots means the Toe. Red ephemeris plot means the satellite is unhealthy. |  |
| Elevation Mask | Set the elevation mask angle (deg) for observation data display. The elevation mask is also used for DOP computation. |  |
| Hide Low Satellite | Set whether low elevation satellites under the elevation mask is shown or not. |  |
| Maximum DOP | Set the y-axis limit of DOP plot. |  |
| Navigation System | Select the navigation systems to be plotted. |  |
| Excluded Sats | Set excluded satellites. Fill in the satellite number or id separated by spaces. |  |
| Error Bar/Circle | Set whether error bar or error circle is shown or not in solution display. |  |
| Direction Arrow | Set whether direction arrow is shown or not in solution ground track display. |  |
| Graph Label | Set whether graph labels are shown or not in solution display. |  |
| Grid Label | Set whether grid labels are shown or not in solution display. |  |
| Compass | Set whether compass is shown or not in solution ground track display. |  |
| Scale | Set whether scale is shown or not in solution ground track display. |  |
| Auto Fit | Set whether scale is adjusted automatically or not. |  |
| Y-Range (+/-) | Set the range of Y-axis in solution display. |  |
| Origin | Select the origin position of solution display. If you select Lat/Lon/Height, you have to input latitude, longitude and ellipsoidal height in the text fields below for the origin. |  |
| Mark Color | Set the mark colors in plots. Click color panel right and select color with color selection dialog. |  |
| Line Color | Set the line color in plots. |  |
| Text Color | Set the text color in plots. |  |
| Grid Color | Set the grid color in plots. |  |
| Background Color | Set the background color in plots. |  |
| Plot Style | Select the plot style in plots. |  |
| Mark Size | Set the mark size in plots. |  |
| Anim. Interval | Set the animation interval for solution or observation data display. |  |
| Font | Select the font in plots. Push ... button and select the font with font selection dialog. |  |

## 3.8	NTRIP Source Table Browser

NTRIP (Networked Transport of RTCM via Internet Protocol) is a communication protocol to interchange GPS/GNSS related data such as receiver raw observation data, ephemerides and corrections for DGPS or RTK-GPS.  NTRIP specifies the table format of so-called Source Table, which represents contents list of provided data by NTRIP servers. RTKLIB includes a simple browser for NTRIP Source Tables.

- Execute the binary AP file rtklib_<ver>\bin\srctblbrows.exe. You can see the main window of NTRIP Source Table Browser.

- Push  button upper left in the main window, leaving right pull down menu of NTRIP caster list blank. If the bottom status bar shows "connecting..." and then "update caster list", the NTRIP caster list is updated. If the pull down menu is blank, the browser acquires the NTRIP caster list from the default NTRIP info caster rtcm-ntrip.org:2101 and update the list. To change the source of the list, fill the NTRIP caster address as the form of <address>:<port> in the pull down menu and push  button. If you omit port number, the browser uses the default port number 2101.

- Select the caster in the pull down menu and push  button. If the status bar shows "source table received", the browser properly received a NTRIP Source Table from the selected NTRIP caster and shows it in the window. The status bar also indicate the error message if a problem arises.

- By pushing field title, you can sort the list. You also can push  STR ,  CAS ,  NET ,  SRC  to switch the contents of the Source Table to Stream List, Caster List, Network List and Original Source Table.

## 3.9	Use Console APs of RTKLIB

RTKLIB includes the following console APs. To execute the APs,

These APs only use standard C functions. So you can rebuild the APs on the non-Windows environment like LINUX, UNIX, MAC OS X and so on. RTKLIB also contains standard make environment for gcc. Refer  Appendix A RTKLIB Console AP Commands for detailed the command line options for these APs.

(1) RTKRCV

Real-time Positioning. The console AP version of RTKNAVI.

(2) RNX2RTKP

Post-Processing Analysis. The console AP version of RTKPOST.

(3) POS2KML

Google Earth KML converter for solution files.

(4) CONVBIN

RINEX Converter of receiver raw data. The console AP version of RTKCONV.

(5) SBSPOS

Positioning with SBAS DGPS corrections.

(6) SBSDUMP

Dump the SBAS correction messages.

(7) STR2STR

Stream Server. Console AP version of STRSVR.

# 4	Develop and Link user AP with RTKLIB

RTKLIB provide the following general purpose C-functions callable from user application program (AP). User can use these function to develop user original positioning APs.

- Matrix and vector functions

- Time and string functions

- Coordinates transformation and geoid model

- Navigation processing

- Positioning models (troposphere, ionosphere, antenna PCV)

- SBAS DGPS/DGNSS correction

- Single point positioning

- Carrier-based and code-based relative positioning

- OTF integer ambiguity resolution

- Receiver raw binary data input

- Positioning solution/NMEA input/output

- RINEX observation data/navigation message input/output

- Precise ephemeris input

- Stream data communication library

- NTRIP (Networked Transport of RTCM via Internet Protocol) library

- RTK-GPS/GNSS positioning server

- RTCM 2.3 and 3.0/3.1 message handling

The following instructions shows the way to utilize the library of RTKLIB in user AP.

- Add the following include directive to the source program of user AP.
#include "rtklib.h"

- Set the following compiler option to add RTKLIB source directory path to compiler include paths.
-I rtklib_<ver>\src

- Add the necessary RTKLIB library source files to source programs set for the AP build. Refer Appendix C Library APIs for detailed library functions and source programs in RTKLIB.

# Appendix A  Console AP Commands

## A.1	RTKRCV

SYNOPSIS

rtkrcv [-s][-p port|-d dev][-o file][-t level]

DESCRIPTION

A command line version of the real-time positioning AP by rtklib. To start or stop RTK server, to configure options or to print solution/status, login a console and input commands. As default, stdin/stdout are used for the console. Use -p option for network login with telnet protocol. To show the available commands, type ? or help on the console. The initial processing options are loaded from default file rtkrcv.conf. To change the file, use -o option. To configure the processing options, edit the options file or use set, load or save command on the console. To shutdown the program, use shutdown command on the console or send USR2 signal to the process.

OPTIONS

-s         start RTK server on program startup

-p port    port number for telnet console

-d dev     terminal device for console

-o file    processing options file

-r level   output solution status file (0:off,1:states,2:residuals)

-t level   debug trace level (0:off,1-5:on)

COMMANDS

start

Start RTK server. No need the command if the program runs with -s option.

stop

Stop RTK server.

restart

Restart RTK server. If the processing options are set, execute the command to enable the changes.

solution [cycle]

Show solutions. Without option, only one solution is shown. With option, the soluiton is displayed at intervals of cycle (s). To stop cyclic display, send break (ctr-C).

status [cycle]

Show RTK status. Use option cycle for cyclic display.

satellite [cycle]

Show satellite status. Use option cycle for cyclic display.

observ [cycle]

Show observation data. Use option cycle for cyclic display.

navidata [cycle]

Show navigation data. Use option cycle for cyclic display.

stream [cycle]

Show stream status. Use option cycle for cyclic display.

error

Show error/warning messages. To stop messages, send break (ctr-C).

option [opt]

Show the values of processing options. Without option, all options are displayed. With option, only pattern-matched options are displayed.

set opt [val]

Set the value of a processing option to val. With out option val, prompt message is shown to input the value. The change of the processing option is not enabled before RTK server is restarted.

load [file]

Load processing options from file. Without option, default file rtkrcv.conf is used. To enable the changes, restart RTK server.

save [file]

Save current processing optons to file. Without option, default file rtkrcv.conf is used.

log [file|off]

Record console log to file. To stop recording the log, use option off.

help|? [path]

Show the command list. With option path, the stream path options are shown.

exit

Exit and logout console. The status of RTK server is not affected by the command.

shutdown

Shutdown RTK server and exit the program.

!command [arg...]

Execute command by the operating system shell. Do not use the interactive command.

NOTES

Short form of a command is allowed. In case of the short form, the command is distinguished according to header characters.rnx2rtkp [option ...]

## A.2	RNX2RTKP

SYNOPSIS

rnx2rtkp [option ...] file file [...]

DESCRIPTION

Read RINEX OBS/NAV files, compute receiver (rover) positions and output position solutions. The first RINEX OBS file shall contain receiver (rover) observations. For the relative mode, the second RINEX OBS file shall contain reference (base) receiver observations. At least one RINEX NAV/GNAV file shall be included in input files. Command options are as follows. ([]:default)

OPTIONS

-h        print help

-o output output file [stdout]

-ts ds ts start day/time (ds=y/m/d ts=h:m:s) [obs start time]

-te de te end day/time   (de=y/m/d te=h:m:s) [obs end time]

-ti tint  time interval (sec) [all]

-p mode   mode (0:single,1:dgps,2:kinematic,3:static,4:moving-base) [2]

-m mask   elevation mask angle (deg) [15]

-f freq   number of frequencies for relative mode (1:L1,2:L1+L2) [2]

-v thres  validation threshold for integer ambiguity (0.0:no AR) [3.0]

-b        backward solutions [off]

-c        forward/backward combined solutions [off]

-i        instantaneous integer ambiguity resolution [off]

-h        fix and hold for integer ambiguity resolution [off]

-e        output x/y/z-ecef position [latitude/longitude/height]

-a        output e/n/u-baseline [latitude/longitude/height]

-n        output NMEA-0183 GGA sentence [latitude/longitude/height]

-g        output latitude/longitude in the form of ddd mm ss.ss' [ddd.ddd]

-z        output height in geodetic height [ellipsoidal]

-t        output time in the form of yyyy/mm/dd hh:mm:ss.ss [sssss.ss]

-u        output time in utc [gpst]

-d col    columns of time under decimal point [3]

-s sep    field separator [' ']

-r x y z  reference (base) receiver ecef pos (m) [average of single pos]

-l lat lon hgt reference (base) receiver latitude/longitude/height (deg/m)

-x level  debug trace level (0:off) [0]

EXAMPLES

Example 1. Kinematic Positioning, L1+L2, output Latitude/Longitude/Height to STDOUT.

> rnx2rtkp 07590920.05o 30400920.05o 30400920.05n

Example 2. Single Point Positioning, El Mask=15deg, output NMEA GGA to file out.pos

> rnx2rtkp -p 0 -m 15 -n -o out.pos 07590920.05o 30400920.05n

Example 3. Static Positioning, L1, time form yyyy/mm/dd hh:mm:ss, output X/Y/Z-ECEF positions

> rnx2rtkp -p 3 -f 1 -t -e 07590920.05o 30400920.05o 30400920.05n

Example 4. Kinematic Positioning, Instantaneous AR, validation threshold=2, comma separator

> rnx2rtkp -i -v 2 -s , 07590920.05o 30400920.05o 30400920.05n

## A.3	POS2KML

SYNOPSIS

pos2kml [option ...] file [...]

DESCRIPTION

Read position file(s) and convert it to Google Earth KML file. Each line in the input file shall contain fields of time, position fields (Latitude/Longitude/Height or X/Y/Z-ECEF), and Quality flag (option). The line started with '%', '#', ';' is treated as comment. Command options are as follows. ([]:default)

OPTIONS

-h        print help

-o file   output file [infile + .kml]

-c color  track color

(0:off,1:white,2:green,3:orange,4:red,5:yellow) [5]

-p color  point color

(0:off,1:white,2:green,3:orange,4:red,5:by qflag) [5]

-a        output altitude information [off]

-ag       output geodetic altitude [off]

-tg       output time stamp of gpst [off]

-tu       output time stamp of utc [gpst]

-i tint   output time interval (s) (0:all) [0]

-q qflg   output q-flags (0:all) [0]

-f n e h  add north/east/height offset to position (m) [0 0 0]

## A.4	CONVBIN

SYNOPSIS

convbin [-ts y/m/d h:m:s] [-te y/m/d h:m:s] [-ti tint] [-r rcv] [-f ffreq]

[-od] [-os] [-d dir] [-o ofile] [-n nfile] [-s sfile] file

DESCRIPTION

Convert receiver raw data log file to RINEX OBS/NAV and SBAS message file. SBAS message file complies to RTKLIB SBAS messsage format. Support the following message formats.

RTCM 2               : Type 1, 3, 9, 14, 16, 17, 18, 19, 22

RTCM 3               : Type 1002, 1004, 1005, 1006, 1010, 1012, 1019, 1020

NovAtel OEMV/4       : RANGECMPB, RANGEB, RAWEPHEMB, IONUTCB, RAWWASSFRAMEB

NovAtel OEM3         : RGEB, RGED, REPB, FRMB, IONB, UTCB

u-blox LEA-4T/AEK-4T : RXM-RAW, RXM-SFRB

NovAtel Superstar II : ID#20, ID#21, ID#22, ID#23, ID#67

Hemisphere           : BIN76, BIN80, BIN94, BIN95, BIN96

SkyTraq S1315F       : msg0xDD, msg0xE0, msg0xDC

OPTIONS

file      input receiver binary log file

-ts y/m/d h:m:s  start time [all]

-te y/m/d h:m:s  end time [all]

-ti tint  observation data interval (s) [all]

-r format log format type

rtcm2= RTCM 2

rtcm3= RTCM 3

nov  = NovAtel OEMV/4

oem3 = NovAtel OEM3

ubx  = ublox AEK-4T(LEA-4T/LEA-5T)

ss2  = NovAtel Superstar II

hemis= Hemisphere Crescent/Eclipse

stq  = SkyTraq S1315F

-f freq   number of frequencies [2]

-od       include doppler frequency [off]

-os       include snr [off]

-d dir    output directory [same as input file]

-o ofile  output RINEX OBS file    [<file>.obs]

-n nfile  output RINEX NAV file    [<file>.nav]

-g gfile  output RINEX GNAV file   [<file>.gnav]

-s sfile  output SBAS message file [<file>.sbs]

If receiver type is not specified, type is recognized by the input file extension as follows.

*.rtcm2  RTCM 2

*.rtcm3  RTCM 3

*.gps    NovAtel OEMV/4

*.ubx    u-blox AEK-4T(LEA-4T)

*.log    NovAtel Superstar II

*.bin    Hemisphere Crescent/Eclipse

*.stq    SkyTraq S1315F

## A.5	SBSPOS

SYNOPSIS

sbspos [option ...] file [...]

DESCRIPTION

Single point positioning with SBAS DGPS corrections. Files shall include receiver RINEX OBS file, NAV file and SBAS message log file (.sbs).

OPTIONS

-o output output file [stdout]

-b        SBAS satellite prn number [129]

-m mask   elevation mask angle (deg) [10]

-c mask   snr mask (dbHz) [0]

-p        single point positioing without SBAS DGPS corrections [off]

-l        apply SBAS long term corrections [all]

-i        apply SBAS ionospheric corrections [all]

-f        apply SBAS fast corrections [all]

-r        apply SBAS ranging [all]

-s        apply doppler smoothing [off]

-t file   output trace to file [off]

## A.6	SBSDUMP

SYNOPSIS

sbsdump [option ...] file

DESCRIPTION

Dump SBAS messages. Specify SBAS log as file.

OPTIONS

-h        print help

-b        sbas satellite prn number [129]

-s        corrected satellite prn number [all]

-f        dump fast correction messages [off]

-i        dump ionospheric correction messages [off]

-l        dump long term correction messages [off]

-n        dump geo navigation message [off]

-g        dump ionospheric grid points [off]

-t        dump integrity messages [off]

# Appendix B  File Formats

## B.1	Positioning Solution File

DESCRIPTION

A positioning solution file is just a text file as output of RTKNAVI or RTKPOST. The file is separated to records or lines by CR/LF. Each records consists of fields. The following table shows the format of the positioning solution file.

| No | Record/Field | Description | Notes |
| --- | --- | --- | --- |
| 1 | File header | The lines starting with "%" are header lines. The header lines contains some additional information or processing options as follows. % program : program version % inp file : Input file path % obs start: Observation data start time in GPS time % obs end: Observation data end time in GPS time % pos mode: Positioning mode option % freqs: Frequencies option % solution: Solution type option % elev mask: Elevation mask angle option % snr mask: SNR mask option % ionos est: Ionospheric parameter estimation option % tropos est: Tropospheric parameters estimation option % amb res: Integer ambiguity resolution options % val thres: Integer ambiguity validation option % ref pos: position of the antenna of the base station |  |
| 2 | Field indicator | Field indicator starting with "%" line follows after File header. To recognize the field formats, RTKLIB uses these lines. Do not delete them. |  |
| 3 | Solution body | Solution body consists of the following fields. The field contents are varied according to the positioning options. |  |
| (1) | Time | The epoch time of the solution indicating the true receiver signal reception time (not indicates the time by receiver clock). The format is varied to the options. yyyy/mm/dd HH:MM:SS.SSS : Calendar time in GPST, UTC or JST, the time system is indicated in Field indicator WWWW SSSSSSS.SSS : GPS week and TOW (time of week) in seconds. |  |
| (2) | Receiver Position | The rover receive antenna or marker position estimated varied according to the positioning options. +ddd.ddddddddd +ddd.dddddddd hhhh.hhhh : Latitude, longitude in degrees and height in m. Minus value means south latitude or west longitude. The height indicates ellipsoidal or geodetic according to the positioning options. +ddd mm ss.sss +ddd mm ss.sss hhhh.hhhh : Latitude, longitude in degree, minute and second and height in m. +xxxxxxxxx.xxxx +yyyyyyyyy.yyyy +zzzzzzzz.zzzz : X/Y/Z components of ECEF frame in m. +eeeeeeeee.eeee +nnnnnnnnn.nnnn +uuuuuuuuu.uuuu : E/N/U components of baseline vector in m. The local coordinate is referenced to the rover position. |  |
| (3) | Quality flag (Q) | The flag which indicates the solution quality. 1 : Fixed, solution by carrier-based relative positioning and the integer ambiguity is properly resolved. 2 : Float, solution by carrier-based relative positioning but the integer ambiguity is not resolved. 3 : Reserved 4 : DGPS, solution by code-based DGPS solutions or single point positioning with SBAS corrections 5 : Single, solution by single point positioning |  |
| (4) | Number of valid satellites (ns) | The number of valid satellites for solution estimation. |  |
| (5) | Standard deviations (sdn, sde, sdu, sdne, sdeu, sdun) | The estimated standard deviations of the solution assuming a priori error model and error parameters by the positioning options. The sdn, sde or sdu means N (north), E (east) or U (up) component of the standard deviations in m. The absolute value of sdne, sdeu or sdun means square root of the absolute value of NE, EU or UN component of the estimated covariance matrix. The sign represents the sign of the covariance. With all of the values, user can reconstruct the full covariance matrix. |  |
| (6) | Age of differential (age) | The time difference between the observation data epochs of the rover receiver and the base station in second. |  |
| (7) | Ratio factor (ratio) | The ratio factor of "ratio-test" for standard integer ambiguity validation strategy. The value means the ratio of the squared sum of the residuals with the second best integer vector to with the best integer vector. |  |

EXAMPLE

% program   : RTKLIB ver.2.3.0b

% inp file  : G:\rtklibtest\20090831\omre196a.09o

% inp file  : G:\rtklibtest\20090831\tevc196a.09o

% inp file  : G:\rtklibtest\20090831\omre196a.09n

% obs start : 2009/07/15 07:10:00.0 GPST (week1540 285000.0s)

% obs end   : 2009/07/15 07:59:50.0 GPST (week1540 287990.0s)

% pos mode  : kinematic

% freqs     : L1+L2

% solution  : forward

% elev mask : 15.0 deg

% snr mask  : 0.0 dBHz

% ionos est : on

% tropo est : on

% amb res   : continuous

% val thres : 3.0

% ref pos   : 32.574831620 -117.126551777   -28.1471

%

% (lat/lon/height=WGS84/ellipsoidal,Q=1:fix,2:float,4:dgps,5:single,ns=# of satellites)

%  GPST                  latitude(deg) longitude(deg)  height(m)   Q  ns   sdn(m)   sde(m)   sdu(m)  sdne(m)  sdeu(m)  sdun(m) age(s)  ratio

2009/07/15 07:10:00.000   32.560273272 -116.953525346   118.6783   1  10   0.0186   0.0202   0.0899  -0.0072   0.0089  -0.0249   0.00    4.5

2009/07/15 07:10:10.000   32.560273266 -116.953525340   118.6877   1  10   0.0144   0.0154   0.0776  -0.0058   0.0082  -0.0199   0.00    5.3

2009/07/15 07:10:20.000   32.560273262 -116.953525365   118.6853   1  10   0.0124   0.0131   0.0720  -0.0051   0.0078  -0.0173   0.00    5.3

2009/07/15 07:10:30.000   32.560273251 -116.953525345   118.6825   1  10   0.0111   0.0117   0.0686  -0.0046   0.0075  -0.0157   0.00    5.6

2009/07/15 07:10:40.000   32.560273275 -116.953525412   118.6827   1  10   0.0103   0.0108   0.0662  -0.0043   0.0073  -0.0146   0.00    4.7

2009/07/15 07:10:50.000   32.560273277 -116.953525429   118.6812   1  10   0.0097   0.0102   0.0644  -0.0041   0.0071  -0.0138   0.00    4.1

2009/07/15 07:11:00.000   32.560273249 -116.953525449   118.6817   1  10   0.0092   0.0097   0.0630  -0.0039   0.0069  -0.0132   0.00    4.2

2009/07/15 07:11:10.000   32.560273271 -116.953525464   118.6729   1  10   0.0088   0.0093   0.0618  -0.0038   0.0067  -0.0127   0.00    5.2

2009/07/15 07:11:20.000   32.560273246 -116.953525468   118.6772   1  10   0.0085   0.0089   0.0607  -0.0037   0.0066  -0.0123   0.00    6.1

2009/07/15 07:11:30.000   32.560273219 -116.953525461   118.6733   1  10   0.0083   0.0087   0.0598  -0.0036   0.0065  -0.0119   0.00    7.9

2009/07/15 07:11:40.000   32.560273216 -116.953525478   118.6771   1  10   0.0081   0.0085   0.0590  -0.0035   0.0064  -0.0117   0.00    9.0

2009/07/15 07:11:50.000   32.560273206 -116.953525489   118.6726   1  10   0.0079   0.0083   0.0582  -0.0035   0.0062  -0.0114   0.00    8.6

2009/07/15 07:12:00.000   32.560273201 -116.953525497   118.6744   1  10   0.0078   0.0081   0.0575  -0.0034   0.0061  -0.0112   0.00    7.5

2009/07/15 07:12:10.000   32.560273212 -116.953525455   118.6731   1  10   0.0077   0.0080   0.0568  -0.0034   0.0060  -0.0110   0.00    7.9

## B.2	SBAS Log File

DESCRIPTION

A SBAS log file is output of RTKCONV, that is a text file in which a line contains a SBAS message captured by the GPS/GNSS receiver. The following table shows the format of the SBAS log file.

| No | Record/Field | Description | Notes |
| --- | --- | --- | --- |
| 1 | SBAS messages | A line contains a SBAS navigation data frame, which consists of the following fields. |  |
| (1) | GPS week number | GPS week number of SBAS navigation data frame. |  |
| (2) | Time of week | Time of week of SBAS navigation data frame in seconds. |  |
| (3) | PRN number | PRN number of SBAS satellite transmitting the navigation data |  |
| (4) | Message type | The type ID of the SBAS message in the frame (0 - 63). Refer SBAS specifications for details: RTCA/DO-229C, Minimum operational performance standards for Global Positioning System/Wide Area Augmentation system airborne equipment |  |
| (5) | Separator | : |  |
| (6) | SBAS message | Hexadecimal dump of a 226-bit SBAS message without 24-bit parity field. The message tail is 0-padded to align to 8-bit boundary. Refer SBAS specifications for the detailed message format. |  |

EXAMPLE

1471 349007 137 25 : C666A0A7F1FE6000027F80000003468000000000000000000000000000

1471 349007 129 25 : C666A0A7F4FE6004047F80000003468000000000000000000000000000

1471 349008 129  4 : 53109FFFFF5FFDFFDFFDFFFFC7FA9FFDFFDFFDFFDFFF9BBBBB33FFFFC0

1471 349008 137  4 : 53129FFC001FFDFFDFFDFFFFA0009FFDFFDFFDFFDFFF9BBBBB33FFFFC0

1471 349009 137  3 : 9A0C9FFDFFDFFFFFDFFC017FF9FFDFFC009FFC015FFFBB97B9BB9FBB80

1471 349009 129  2 : 9A0A9FFFFC9FFFFE9FFDFFDFFDFFDFFDFFFFF7F93FFBE79BBBBBB9FA00

1471 349010 137  2 : C60A9FFFFD1FFFFFDFFDFFDFFDFFDFFDFFC003F88003E79FBBFBB9FA00

1471 349010 129  3 : C60E9FFDFFDFFFFE9FFC007FEDFFDFFFFFDFFFFFDFFFBB97B9BB9FBB80

1471 349011 137 26 : 536A0029E0EF0FF05F829C11C076033015A09D047037C1DE14F08FE000

1471 349011 129 26 : 536A0821A0DD05E82B813E0EF0F7897C27C12E08683B419C0BE057E000

1471 349012 137 28 : 9A723440E44E810029FF1F1F379C0BC35D4BE2B8078F15903253960200

1471 349012 129 28 : 9A722CB5D8739087B46B107DA8D9E828694B55F843782100AF146AD980

1471 349013 129  9 : C62434198D3F5D92BA855704800236DFE84FE06FFA47FE7FF0008E0240

1471 349013 137  9 : C6260C198D32310732404C1D40183CDFD187C8F3FF7FFD800FF7D6BE40

1471 349014 129  4 : 53119FFFFF9FFDFFDFFDFFFFD3FA5FFDFFDFFDFFDFFF9BBBBB33FFFFC0

1471 349014 137  4 : 53109FFC005FFDFFDFFDFFFFAFFFDFFDFFDFFDFFDFFF9BBBBB33FFFFC0

1471 349015 129  2 : 9A089FFFFC5FFFFEDFFDFFDFFDFFDFFDFFFFFBF8FFFFE79BBBBBB9FA00

1471 349015 137  3 : 9A0D9FFDFFDFFFFF9FFC017FFDFFDFFC00DFFC015FFFBB97B9BB9FBB80

1471 349016 137  2 : C6089FFFFD5FFC001FFDFFDFFDFFDFFDFFFFFFF8BFFFE79FBBFBB9FA00

1471 349016 129  3 : C60C9FFDFFDFFFFE5FFC007FF1FFDFFC001FFFFFDFFFBB97B9BB9FBB80

1471 349017 137 25 : 5366587803FE3FF0010080FFFF835E8000000000000000000000000000

1471 349017 129 25 : 5366587FFDFEDFF40400800000035E8000000000000000000000000000

1471 349018 129 63 : 9AFC000000000000000000000000000000000000000000000000000000

1471 349018 137 63 : 9AFC000000000000000000000000000000000000000000000000000000

1471 349019 129 26 : C66A0C53E26F09704781DC0DE06702FC19E1EF09F047821C0EF05FE000

1471 349019 137 26 : C66A0431E17E0A704741F80DC05E827815C0EF09F877C2FC15E096E000

1471 349020 137  4 : 53119FFC001FFDFFDFFDFFFF9C001FFDFFDFFDFFDFFF9BBBBB33FFFFC0

1471 349020 129  4 : 53129FFFFF5FFDFFDFFDFFFFC3FA9FFDFFDFFDFFDFFF9BBBBB33FFFFC0

## B.3	Solution Status File

DESCRIPTION

A solution status file is output of RTKNAVI or RTKPOST, that is a text file which contains the internal status of the positioning process. The internal status include estimated states of Kalman filter and residuals of measurements to analyze the solution quality. The following table shows the format of the solution status file.

| No | Record/Field | Description | Notes |
| --- | --- | --- | --- |
| 1 | Position States | Estimated rover position in the filter. The format of a record is as follows. $POS,week,tow,stat,posx,posy,posz,posxf,posyf,poszf week/tow : gps week no/time of week (s) stat : solution status posx/posy/posz : position x/y/z ecef (m) float posxf/posyf/poszf : position x/y/z ecef (m) fixed |  |
| 2 | Velocity/ Acceleration States | Estimated rover velocity and acceleration in the filter. The format of a record is as follows. $VELACC,week,tow,stat,vele,veln,velu,acce,accn,accu,velef,velnf,\ veluf,accef,accnf,accuf week/tow : gps week no/time of week (s) stat : solution status vele/veln/velu : velocity e/n/u (m/s) float acce/accn/accu : acceleration e/n/u (m/s^2) float velef/velnf/veluf : velocity e/n/u (m/s) fixed accef/accnf/accuf : acceleration e/n/u (m/s^2) fixed |  |
| 3 | Ionosphere Parameter States | Estimated ionosphere parameter (vertical L1 ionosphere delay difference). The format of a record is as follows. $ION,week,tow,stat,sat,az,el,ion,ion-fixed week/tow : gps week no/time of week (s) stat : solution status sat : satellite id az/el : azimuth/elevation angle(deg) ion : vertical ionospheric delay L1 (m) float ion-fixed: vertical ionospheric delay L1 (m) fixed |  |
| 4 | Troposphere Parameter States | Estimated troposphere parameter (vertical troposphere delay residual). The format of a record is as follows. $TROP,week,tow,stat,rcv,ztd,ztdf week/tow : gps week no/time of week (s) stat : solution status rcv : receiver (1:rover,2:base station) ztd : zenith total delay (m) float ztdf : zenith total delay (m) fixed |  |
| 5 | Receiver H/W bias States | Estimated GLONASS receiver H/W bias difference. The format of a record is as follows. $HWBIAS,week,tow,stat,frq,bias,biasf week/tow : gps week no/time of week (s) stat : solution status frq : frequency (1:L1,2:L2,...) bias : h/w bias coefficient (m/MHz) float biasf : h/w bias coefficient (m/MHz) fixed |  |
| 6 | Residuals | Residuals of pseudorange and carrier-phase observables. The format of a record is as follows. $SAT,week,tow,sat,frq,az,el,resp,resc,vsat,snr,fix,slip,lock,outc,\ slipc,rejc week/tow : gps week no/time of week (s) sat/frq : satellite id/frequency (1:L1,2:L2,...) az/el : azimuth/elevation angle (deg) resp : pseudorange residual (m) resc : carrier-phase residual (m) vsat : valid data flag (0:invalid,1:valid) snr : signal strength (dbHz) fix : ambiguity flag (0:no data,1:float,2:fixed,3:hold) slip : cycle-slip flag (bit1:slip,bit2:parity unknown) lock : carrier-lock count outc : data outage count slipc : cycle-slip count rejc : data reject (outlier) count |  |

EXAMPLE

$POS,1557,432000.000,2,-3869295.9628,3436570.2567,3717367.6546,0.0000,0.0000,0.0000

$HWBIAS,1557,432000.000,2,1,-0.3503,0.0000

$HWBIAS,1557,432000.000,2,2,0.0108,0.0000

$SAT,1557,432000.000,3,1,253.2,64.3,0.3219,-0.0006,1,48,1,1,1,0,1,0

$SAT,1557,432000.000,3,2,253.2,64.3,-0.0629,-0.0006,1,33,1,1,1,0,1,0

$SAT,1557,432000.000,13,1,298.4,24.1,-0.6732,0.0003,1,42,1,1,1,0,1,0

$SAT,1557,432000.000,13,2,298.4,24.1,0.8081,0.0003,1,17,1,1,1,0,1,0

$SAT,1557,432000.000,16,1,42.0,59.5,0.5037,-0.0005,1,47,1,1,1,0,1,0

$SAT,1557,432000.000,16,2,42.0,59.5,-0.5170,-0.0005,1,30,1,1,1,0,1,0

$SAT,1557,432000.000,19,1,229.8,39.0,-0.1948,-0.0003,1,44,1,0,1,0,0,0

$SAT,1557,432000.000,19,2,229.8,39.0,-0.0806,-0.0003,1,28,1,1,1,0,1,0

$SAT,1557,432000.000,21,1,61.1,28.1,-1.0704,0.0001,1,41,1,1,1,0,1,0

$SAT,1557,432000.000,21,2,61.1,28.1,1.0139,0.0001,1,19,1,1,1,0,1,0

$SAT,1557,432000.000,23,1,257.9,29.9,-1.3258,-0.0000,1,45,1,1,1,0,1,0

$SAT,1557,432000.000,23,2,257.9,29.9,0.4155,0.0000,1,23,1,1,1,0,1,0

$SAT,1557,432000.000,25,1,317.0,24.7,0.8868,0.0002,1,41,1,1,1,0,1,0

$SAT,1557,432000.000,25,2,317.0,24.7,0.1811,0.0003,1,19,1,1,1,0,1,0

$SAT,1557,432000.000,31,1,145.1,32.5,0.6140,-0.0001,1,44,1,1,1,0,1,0

$SAT,1557,432000.000,31,2,145.1,32.5,-0.2397,-0.0001,1,26,1,1,1,0,1,0

$SAT,1557,432000.000,R9,1,105.7,78.1,-0.1172,-0.0001,1,45,1,1,1,0,1,0

$SAT,1557,432000.000,R9,2,105.7,78.1,0.0000,0.0000,0,0,0,0,0,1,0,0

$SAT,1557,432000.000,R10,1,331.5,41.7,-0.1425,0.0002,1,43,1,1,1,0,1,0

$SAT,1557,432000.000,R10,2,331.5,41.7,0.0349,0.0001,1,30,1,1,1,0,1,0

$SAT,1557,432000.000,R19,1,18.6,61.2,-0.7708,-0.0000,1,46,1,1,1,0,1,0

$SAT,1557,432000.000,R19,2,18.6,61.2,0.1898,-0.0001,1,39,1,0,1,0,0,0

$SAT,1557,432000.000,R20,1,235.7,55.6,1.0305,-0.0000,1,42,1,1,1,0,1,0

$SAT,1557,432000.000,R20,2,235.7,55.6,-0.2247,-0.0001,1,39,1,1,1,0,1,0

# Appendix C  Library APIs (Application Program Interfaces)

The following table shows the list of RTKLIB library functions. For detailed API (calling convention, description of the function, input and output parameters and types, return value and type) for a library function, refer the header comment of each function in the source program in rtklib_<ver>\src. The definition of data types regarding to the APIs, refer the header file rtklib.h in rtklib_<ver>\src.

RTKLIB API FUNCTION LIST

| Function | Description | Source Program |
| --- | --- | --- |
|  | Satellite number/system functions |  |
| satno() | Satellite system and PRN/slot number to satellite number | rtkcmn.c |
| satsys() | Satellite number to satellite system | rtkcmn.c |
| satid2no() | Satellite ID to satellite number | rtkcmn.c |
| satno2id() | Satellite number to satellite ID | rtkcmn.c |
|  | Matrix and vector functions |  |
| mat() | New matrix | rtkcmn.c |
| imat() | New integer matrix | rtkcmn.c |
| zeros() | New zero matrix | rtkcmn.c |
| eye() | New identity matrix | rtkcmn.c |
| dot() | Inner Product | rtkcmn.c |
| norm() | Euclid norm | rtkcmn.c |
| matcpy() | Copy matrix | rtkcmn.c |
| matmul() | Multiply matrix | rtkcmn.c |
| matinv() | Inverse of matrix | rtkcmn.c |
| solve() | Solve linear equation | rtkcmn.c |
| lsq() | Least square estimation | rtkcmn.c |
| filter() | Kalman filter state update | rtkcmn.c |
| smoother() | Kalman smoother | rtkcmn.c |
| matprint() | Print matrix | rtkcmn.c |
| matfprint() | Print matrix to file | rtkcmn.c |
|  | Time and string functions |  |
| str2num() | String to number | rtkcmn.c |
| str2time() | String to time | rtkcmn.c |
| time2str() | Time to string | rtkcmn.c |
| epoch2time() | Calendar day/time to time | rtkcmn.c |
| time2epoch() | Time to calendar day/time | rtkcmn.c |
| gpst2time() | GPSTIME to time | rtkcmn.c |
| time2gpst() | Time to GPSTIME | rtkcmn.c |
| timeadd() | Add time | rtkcmn.c |
| timediff() | Time difference | rtkcmn.c |
| gpst2utc() | GPSTIME to UTC | rtkcmn.c |
| utc2gpst() | UTC to GPSTIME | rtkcmn.c |
| timeget() | Get current time in UTC | rtkcmn.c |
| time2doy() | Time to Day of Year | rtkcmn.c |
| adjgpsweek() | Adjust gps week number | rtkcmn.c |
| tickget() | Get current tick time | rtkcmn.c |
| sleepms() | Sleep for milli-seconds | rtkcmn.c |
|  | Coordinates functions |  |
| ecef2pos() | ECEF to geodetic position | rtkcmn.c |
| pos2ecef() | Geodetic to ECEF position | rtkcmn.c |
| ecef2enu() | ECEF to local coordinates | rtkcmn.c |
| enu2ecef() | Local to ECEF coordinates | rtkcmn.c |
| covenu() | Covariance in local coordinates | rtkcmn.c |
| covecef() | Covariance in ECEF coordinates | rtkcmn.c |
| xyz2enu() | ECEF to ENU local coordinate transformation matrix | rtkcmn.c |
|  | Input/output functions |  |
| readpcv() | Read antenna phase center parameters | rtkcmn.c |
| readpos() | Read station positions | rtkcmn.c |
| sortobs() | Sort observation data | rtkcmn.c |
| uniqeph() | Delete duplicated ephemeris | rtkcmn.c |
| screent() | Screen data by time and interval | rtkcmn.c |
| readnav() | Read navigation data from file | rtkcmn.c |
| savenav() | Save navigation data to file | rtkcmn.c |
|  | Debug trace functions |  |
| traceopen() | Open trace file | rtkcmn.c |
| traceclose() | Close trace file | rtkcmn.c |
| trace() | Output trace | rtkcmn.c |
| tracet() | Output trace with time tag | rtkcmn.c |
| tracemat() | Output trace as matrix printing | rtkcmn.c |
| traceobs() | Output trace as observation data printing | rtkcmn.c |
| traceonav() | Output trace as GPS navigation messages printing | rtkcmn.c |
| tracegnav() | Output trace as GLONASS navigation messages | rtkcmn.c |
| tracepeph() | Output trace as precise ephemrides | rtkcmn.c |
| traceb() | Output trace as binary dump | rtkcmn.c |
|  | Platform dependent functions |  |
| execcmd() | Execute command | rtkcmn.c |
| expath() | Expand file path | rtkcmn.c |
|  | Positioning models |  |
| eph2pos() | GPS ephemeris to satellite position/clock-bias | rtkcmn.c |
| geph2pos() | GLONASS ephemeris to satellite position/clock-bias | rtkcmn.c |
| satpos() | Satellite positions/clock-biases | rtkcmn.c |
| satposv() | Satellite positions/velocities/clock-biases/clock-drifts | rtkcmn.c |
| satposiode() | Satellite positions/clock-biases by IODE * | rtkcmn.c |
| satwavelen() | Satellite signal carrier wave length | rtkcmn.c |
| satazel() | Satellite azimuth/elevation angle | rtkcmn.c |
| geodist() | Geometric distance | rtkcmn.c |
| dops() | Compute DOPs | rtkcmn.c |
| ionmodel() | Ionospheric model | rtkcmn.c |
| ionmapf() | Ionospheric mapping function | rtkcmn.c |
| tropmodel() | Tropospheric model | rtkcmn.c |
| tropmapf() | Tropospheric mapping function (NMF) | rtkcmn.c |
| antmodel() | Antenna model | rtkcmn.c |
| csmooth() | Carrier smoothing | rtkcmn.c |
|  | Single-point positioning |  |
| pntpos() | Single-point positioning | rtkcmn.c |
| pntvel() | Velocity estimation by Single-point positioning | rtkcmn.c |
|  | Geoid model |  |
| opengeoid() | Open external geoid file | geoid.c |
| closegeoid() | Close external geoid file | geoid.c |
| geoidh() | Geoid height | geoid.c |
|  | Datum transformation |  |
| loaddatump() | Load datum transformation parameter | datum.c |
| tokyo2jgd() | Tokyo datum to JGD2000 datum | datum.c |
| jgd2tokyo() | JGD2000 datum to Tokyo datum | datum.c |
|  | RINEX functions |  |
| readrnx() | Read RINEX files | rinex.c |
| readrnxt() | Read RINEX files in time range/interval | rinex.c |
| outrnxobsh() | Output RINEX OBS header | rinex.c |
| outrnxobsb() | Output RINEX OBS body | rinex.c |
| outrnxnavh() | Output RINEX NAV header | rinex.c |
| outrnxnavb() | Output RINEX NAV body | rinex.c |
| outrnxgnavh() | Output RINEX GNAV header | rinex.c |
| outrnxgnavb() | Output RINEX GNAV body | rinex.c |
| uncompress() | Uncompress file | rinex.c |
| convrnx() | RINEX converter | convrnx.c |
|  | Precise ephemeris functions |  |
| readsp3() | Read SP3 file | preceph.c |
| readsap() | Read satellite antenna phase center position | preceph.c |
| eph2posp() | Satellite precise ephemeris to satellite position/clock-bias | preceph.c |
| sat2posp() | Satellite positions/clock-biases with precise ephemeris | preceph.c |
|  | Receiver raw data functions |  |
| getbitu() | Extract unsigned bits | rtkcmn.c |
| getbits() | extract signed bits | rtkcmn.c |
| crc32() | CRC32 parity | rtkcmn.c |
| crc24q() | CRC24Q parity | rtkcmn.c |
| decode_word() | Decode navigation data word | rcvraw.c |
| decode_frame() | Decode navigation data frame | rcvraw.c |
| init_raw() | Initialize receiver raw data control | rcvraw.c |
| free_raw() | Free receiver raw data control | rcvraw.c |
| input_raw() | Input receiver raw data from stream | rcvraw.c |
| input_rawf() | Input receiver raw data from file | rcvraw.c |
|  | Receiver dependent functions |  |
| input_oem4() | Input OEM4/V raw data from stream | rcv/novatel.c |
| input_oem3() | Input OEM3 raw data from stream | rcv/novatel.c |
| input_ubx() | Input u-blox raw data from stream | rcv/ublox.c |
| input_ss2() | Input Superstar II raw data from stream | rcv/ss2.c |
| input_cres() | Input Crescent raw data from stream | rcv/crescent.c |
| input_oem4f() | Input OEM4/V raw data from file | rcv/novatel.c |
| input_stq() | Input SkyTraq raw data from stream | rcv/skytraq.c |
| input_oem3f() | Input OEM3 raw data from file | rcv/novatel.c |
| input_ubxf() | Input u-blox raw data from file | rcv/ublox.c |
| input_ss2f() | Input Superstar II raw data from file | rcv/ss2.c |
| input_cresf() | Input Crescent raw data from file | rcv/crescent.c |
| input_stqf() | Input SkyTraq raw data from file | rcv/skytraq.c |
| gen_ubx() | Generate u-blox binary command | rcv/ublox.c |
| gen_stq() | Generate SkyTraq binary command | rcv/skytraq.c |
|  | RTCM functions |  |
| init_rtcm() | Initialize RTCM control | rtcm.c |
| free_rtcm() | Free RTCM control | rtcm.c |
| input_rtcm2() | Input RTCM2 message from stream | rtcm.c |
| input_rtcm3() | Input RTCM3 message from stream | rtcm.c |
| input_rtcm2f() | Input RTCM2 message from file | rtcm.c |
| input_rtcm3f() | Input RTCM3 message from file | rtcm.c |
|  | Solution functions |  |
| readsol() | Read solutions | solution.c |
| readsolt() | Read solutions in time range/interval | solution.c |
| readsolstat() | Read solution status file | solution.c |
| outsolheads() | Output solution header to string | solution.c |
| outsols() | Output solution body to string | solution.c |
| outsolexs() | Output extended solution to string | solution.c |
| outsolhead() | Output solution header to file | solution.c |
| outsol() | Output solution body to file | solution.c |
| outsolex() | Output extended solution to file | solution.c |
| setsolopt() | Set solution output options | solution.c |
| setsolformat() | Set solution output format | solution.c |
| outnmea_rmc() | Set solution output format | solution.c |
| outnmea_gga() | Set solution output format | solution.c |
| outnmea_gsa() | Set solution output format | solution.c |
| outnmea_gsv() | Set solution output format | solution.c |
|  | Convert solutions to Google Earth KML file |  |
| convkml() | Convert solutions to Google Earth KML file | convkml.c |
|  | SBAS functions |  |
| sbsreadmsg() | Read SBAS message file | sbas.c |
| sbsreadsmgt() | Read SBAS message file in time range | sbas.c |
| sbsoutmsg() | Output SBAS messages | sbas.c |
| sbsupdatestat() | Update SBAS status | sbas.c |
| sbsdecodemsg() | Decode SBAS message | sbas.c |
| sbssatpos() | SBAS satellite position | sbas.c |
| sbspntpos() | SBAS point positioning with corrections | sbas.c |
|  | Integer least-square estimation |  |
| lambda() | LAMBDA/MLAMBDA integer least-square estimation | lambda.c |
|  | Real-time kinematic positioning |  |
| rtkinit() | Initialize RTK control struct | rtkpos.c |
| rtkfree() | Free RTK control struct | rtkpos.c |
| rtkpos() | RTK positioning | rtkpos.c |
| rtkopenstat() | Open solution status file | rtkpos.c |
| rtkclosestat() | Close solution status file | rtkpos.c |
|  | Post-processing positioning |  |
| postpos() | Post-processing positioning | postpos.c |
| postposopt() | Set post-processing positioning options | postpos.c |
| readopts() | Read positioning options | postpos.c |
| writeopts() | Write positioning options | postpos.c |
|  | Stream data input/output functions |  |
| strinitcom() | Initialize stream communication environment | stream.c |
| strinit() | Initialize stream | stream.c |
| strlock() | Lock stream | stream.c |
| strunlock() | Unlock stream | stream.c |
| stropen() | Open stream | stream.c |
| strclose() | Close stream | stream.c |
| strread() | Read stream | stream.c |
| strwrite() | Write stream | stream.c |
| strsync() | Time sync stream | stream.c |
| strstat() | Get stream status | stream.c |
| strsum() | Get stream statistics summary | stream.c |
| strsetopt() | Set stream options | stream.c |
| strgettime() | Get current time from stream | stream.c |
| strsendnmea() | Send NMEA message to stream | stream.c |
| strsendcmd() | Send receiver command to stream | stream.c |
|  | Stream server functions |  |
| strsvrinit() | Initialize stream server | stream.c |
| strsvrstart() | Start stream server | stream.c |
| strsvrstop() | Stop stream server | stream.c |
| strsvrstat() | Get stream server status | stream.c |
|  | RTK server functions |  |
| rtksvrinit() | Initialize RTK server | rtksvr.c |
| rtksvrstart() | Start RTK server | rtksvr.c |
| rtksvrstop() | Stop RTK server | rtksvr.c |
| rtksvrlock() | Lock RTK server | rtksvr.c |
| rtksvrunlock() | Unlock RTK server | rtksvr.c |
| rtksvrostat() | Get RTK observation data status | rtksvr.c |
| rtksvrsstat() | Get RTK stream status | rtksvr.c |